# HYKON Firmware Updater

User Manual

Version: 1.1

Release Date: 16 April 2026

## Table of Contents

- Introduction

- System Requirements

- Hardware Setup & Connections

- USB CAN Waveshare Driver Installation

- Getting Started

- Main Features

- Step‑by‑Step Instructions

- Troubleshooting

- Safety Warnings

- Technical Specifications

- FAQ

- Contact & Support

- Version History

# 1. Introduction

HYKON Firmware Updater is a professional ECU / Instrument Cluster maintenance tool designed for authorized automotive technicians and service centers.

The application supports three core operations:

## 1.1 Firmware Update

- Updates cluster firmware and assets

- Cluster must be physically removed from vehicle

- Requires external 12V power supply

- Uses USB CAN Waveshare adapter

## 1.2 Service Reset

- Clears service warnings & maintenance indicators

- Cluster remains installed in vehicle

- Connects via vehicle CAN bus

## 1.3 Odometer Reset

- Resets odometer and trip data

- Cluster remains installed in vehicle

- Password‑protected

# 2. System Requirements

## 2.1 Hardware Requirements

Firmware Update Mode

- USB CAN Waveshare adapter

- External 12V DC power supply (≥ 2A)

- Jumper wires (22‑AWG recommended)

- Cluster pin extractor tool

Service Reset / Odometer Reset

- USB CAN Waveshare adapter

- CAN bus tap probe or OBD‑II connector

- Jumper wires (22‑AWG)

## 2.2 Software Requirements

- Windows 10 or Windows 11

- CH341SER driver driver

- Administrator privileges

## 2.3 Supported Vehicles

- Hykon Hetto Plus

# 3. Hardware Setup & Connections

## 3.1 Firmware Update (Bench Programming)

⚠ IMPORTANT: Cluster MUST be removed from the vehicle

#### Required Components

- Removed instrument cluster

- 12V power supply

- USB CAN Waveshare adapter

- CANH / CANL jumper wires

#### Connection Summary

- VCC (RED) → Cluster 12V pin

- GND (BLACK) → Cluster ground pin

- CANH (WHITE) → Adapter CANH

- CANL (YELLOW) → Adapter CANL

Power ON sequence:

- Connect all wiring

- Plug USB adapter into PC

- Power ON 12V supply

- Launch HYKON application

## 3.2 Service Reset & Odometer Reset (In‑Vehicle)

✅ Cluster removal NOT required

#### Recommended Connection

- OBD‑II connector

  - Pin 6 → CANH

  - Pin 14 → CANL

  - Pin 4/5 → GND

Vehicle ignition must be ACC or ON.

# 4. USB CAN Waveshare Driver Installation

## 4.1 Automatic Installation (Recommended)

- Download the CH341SER driver from Waveshare’s official site.

https://files.waveshare.com/upload/c/c4/CH341SER.zip

- Run CH341SER.EXE as Administrator.

- When “Driver Installation Successful” appears, click OK.

- Restart your PC.

- Connect the USB-CAN-A adapter.

- Open Device Manager → Ports (COM & LPT) and verify the adapter appears as a COM port (e.g., COM3).

## 4.2 Verification

- The adapter should be visible under Ports with no warning symbols.

- Default serial baud rate is 2 Mbps (2000000 bps).

- Use the USB-CAN-A Configuration Tool (USB-CAN.exe) to open the COM port and configure CAN parameters (baud rate, mode, filters).

# 5. Getting Started

- Extract HYKON application folder

- Double-click Hykon.exe

- Mode selection screen appears

# 6. Main Features

### Firmware Update

- COM port auto‑detection

- Firmware & Assets update

- Progress tracking

- Error detection

### Service Reset

- Clears maintenance warnings

- Fast operation (1–2 seconds)

### Odometer Reset

- Secure password protection

- Permanent reset

# 7. Step‑by‑Step Instructions

## 7.1 Firmware Update Procedure

- Remove cluster from vehicle

- Connect power and CAN wiring

- Launch application

- Select Firmware Update and select the COM port

- Under the update type you will see two options

##### A. Firmware Update

- Click Firmware Update.

- Click Browse File and select the firmware.bin file.

- Click START to begin the update.

- Verify that:

- The progress bar starts incrementing within 10 sec.

- The cluster display shows FLASHING, indicating the update has started.

- Wait until the message “Update Successfully Completed” appears.

##### B. Asset Update

- Click Asset Update.

- Click Browse File and select the asset.bin file.

- Click START to begin the update.

- Verify that:

- The progress bar starts incrementing within 20 sec.

- The cluster display shows FLASHING, indicating the update has started.

- Wait until the message “Update Successfully Completed” appears.

Note:
 If the progress bar does not start incrementing or the cluster does not show FLASHING within the expected time, the cluster screen turns red, if the update gets interrupted between or any other pop-up message appears. Refer to the Troubleshooting section.

## B. Service Reset Procedure

- Connect USB CAN to vehicle

- Launch application

- Select Service Reset

- Choose COM port

- Start reset

- Confirm success

## C. Odometer Reset Procedure

- Same setup as Service Reset

- Select Odometer Reset

- Enter password: hykon123

- Confirm reset

⚠ Operation is irreversible

Note:
 If the service warning or odometer value is not cleared. Refer to the Troubleshooting section.

# 8. Troubleshooting

## 8.1 How to verify that connection is established

- Open the HYKON application.

- Click on CONNECT.

- Observe the cluster display:

- If the display turns OFF and then turns ON immediately, the connection has been successfully established.

- If the display does not reset as described above, check the CAN wiring and power connections

- If the display stays in the cyan screen for 20 sec or more, that means an update is pending.

## 8.2 Common Issues

| Issue | Solution |
| --- | --- |
| No COM ports | Reinstall driver, reconnect USB |
| No communication | Check CANH / CANL wiring |
| Driver blocked | Run installer as Administrator |

## 8.3 Issues with Firmware update

### 8.3.1 Pop-up messages

#### File error

- If this message comes up you haven't selected a binary file. Click on browse file to select the binary file

#### Version mismatch or Connection Error

- If this comes up first, verify the connection using step 8.2

- If the connection is successful, you are using an older firmware version, use a newer one

#### Connection error

- If this comes up the connection between laptop and USB CAN Waveshare is lost.

- Realaunch the application

- Turn off the cluster, turn it on after a second

- If the cluster screen is cyan color click start again.

- If the cluster goes black, again restart

#### Update Failure

- If this comes up the update did not happen correctly.

- Realaunch the application

- Turn off the cluster, turn it on after a second

- Once the cluster screen is cyan color click start again.

- If the cluster goes black, again restart

#### COM port Busy

### 8.3.2 Display anomalies

#### Cyan Screen

- You are stuck in bootloader

- Click firmware update, select binary file and click start

- If nothing happens restart and try again

#### Red Screen

- Means you missed a frame in between

- Restart the cluster and you will end up in the cyan screen

#### Flashing Screen

- If u are stuck in this screen for more than 5 minutes restart the custer

- You will reach the cyan screen

#### Black sceen

- Change the cluster

- Plainning for something else

#### Special Case

- This happens when the assets are not updated, do the asset update.

## 9. Safety Warnings

⚠ CRITICAL WARNINGS

- Never disconnect power during update

- Never remove cluster with battery connected

- Use stable automotive‑grade power supply

- Maintain ESD protection

## 10. Technical Specifications

- Protocol: CAN 2.0B (Extended)

- Baud Rate: 2 Mbps

- File Format: .bin

- OS: Windows 10 / 11

## 11. FAQ

Q: Is cluster removal mandatory for firmware update?

A: Yes. Bench programming requires isolated power.

Q: Can update be reversed?

A: No. Firmware overwrite is irreversible.

Q: Is odometer reset legal?

A: Depends on local law. Authorization required.

## 12. Contact & Support

For support, provide:

- Windows version

- COM port number

- Cluster model

- Error message (if any)

## 13. Version History

| Version | Date | Notes |
| --- | --- | --- |
| 1.0 | 16‑Apr‑2026 | Initial Release |
| 1.1 | 20‑Apr‑2026 | Documentation Improvements |

© HYKON – Authorized Service Tool